MICROPROCESSOR & MICROCONTROLLER (BE05041011)  ·  B.E. SEMESTER 5

Laboratory Quiz / Viva Answers

Short answers to the quiz questions of all 12 experiments — for hand-written submission

| Particulars | Details |
| --- | --- |
| Name of the student |  |
| Enrolment No. · Branch | Robotics and Automation |
| Subject | Microcontroller and Microprocessor (BE05041011) |
| Experiments covered | All 12 (8085 · 8051 · AVR) |
| Date |  |

# Experiment 1 — Introduction to Microprocessor (8085)

1.  What are the various registers available in 8085?

Ans.  The Accumulator (A); six general-purpose registers B, C, D, E, H, L (used singly or as the pairs BC, DE, HL); the Flag register; and two 16-bit registers — the Program Counter (PC) and Stack Pointer (SP).

2.  What are the various flags used in 8085?

Ans.  Five flags — Sign (S), Zero (Z), Auxiliary Carry (AC), Parity (P) and Carry (CY) — which indicate the status of the result of an ALU operation.

3.  What is stack pointer?

Ans.  A 16-bit register that holds the address of the top of the stack — a LIFO area of memory used to store return addresses and register data during PUSH, POP, CALL and RET.

4.  What is program counter?

Ans.  A 16-bit register that holds the address of the next instruction to be fetched; it is incremented automatically so the program executes in sequence.

5.  Define interrupt.

Ans.  A signal that halts the main program and transfers control to a service routine (ISR); after the ISR finishes, control returns to the main program.

6.  What are the hardware & software interrupts?

Ans.  Hardware interrupts come from external pins — TRAP, RST 7.5, RST 6.5, RST 5.5, INTR. Software interrupts are produced by the instructions RST 0 to RST 7 written in the program.

7.  What is the clock frequency for 8085?

Ans.  About 3 MHz internally. A 6 MHz crystal is connected across X1–X2 and is divided by 2 on-chip to give the operating frequency.

8.  What is ALE? Explain the function of ALE in 8085.

Ans.  ALE = Address Latch Enable. It is used to demultiplex the AD0–AD7 bus: when ALE = 1 the lines carry the lower address byte (latched into an external latch), and afterwards the same lines carry data.

# Experiment 2 — Data Transfer Instructions

1.  Write the names of addressing modes.

Ans.  The 8085 has five — Immediate, Direct, Register, Register Indirect and Implied (Implicit).

2.  List any three data transfer instructions with immediate addressing mode.

Ans.  MVI A, 05H  ·  MVI B, 0FH  ·  LXI H, 2000H — the data is given directly in the instruction.

3.  Explain the operation of PUSH & POP instruction with a suitable program.

Ans.  PUSH stores a register pair on the stack (SP − 2); POP retrieves a pair from the stack (SP + 2), on a LIFO basis.  e.g. LXI SP,27FFH → PUSH B (save BC) → POP D (load DE from stack).

# Experiment 3 — Arithmetic and Logical Instructions

1.  What is the purpose of the ADD instruction in 8085?

Ans.  It adds the contents of a register/memory to the accumulator and stores the result in A (A = A + R); all flags are affected.

2.  Which 8085 instruction is used for logical AND operations?

Ans.  ANA (with a register/memory) and ANI (with immediate data); mainly used to mask bits.

3.  Which instruction decrements a register or memory location?

Ans.  DCR (e.g. DCR B, DCR M) for an 8-bit register/memory; DCX is used for a 16-bit register pair.

4.  What is the role of the CMP instruction?

Ans.  It compares a register/memory with the accumulator by subtraction; the result is not stored — only the flags are set (Z if equal, CY if A is smaller).

5.  Which instruction is used to clear a specific bit in a register/memory?

Ans.  Use ANI/ANA with a mask having 0 at the bit to be cleared and 1 elsewhere — e.g. ANI 0FEH clears bit 0.

6.  What is the purpose of the XRA instruction?

Ans.  It performs a bitwise Exclusive-OR of A with a register/memory. XRA A is commonly used to clear the accumulator to 00H.

7.  Which instruction performs addition with carry?

Ans.  ADC (register/memory) and ACI (immediate) — they add the operand plus the carry to A, and are used in multi-byte addition.

# Experiment 4 — Branching and Looping Instructions

1.  What is the difference between conditional & unconditional branching instructions?

Ans.  Unconditional branches always transfer control (JMP, CALL, RET). Conditional branches transfer control only if a flag condition is satisfied (JZ, JNZ, JC, JNC, JP, JM, etc.).

2.  What is the difference in operation of JMP & CALL instruction?

Ans.  JMP transfers control to an address without saving any return address. CALL first saves the return address on the stack and then jumps, so a RET can return to the main program.

# Experiment 5 — Data Transfer Programming in 8051

1.  List the available software for the microcontroller practical.

Ans.  Keil µVision, SimulIDE, Proteus, MIDE-51 and EdSim51.

2.  Describe the dissimilarities between internal and external memory in the 8051.

Ans.  Internal memory is on-chip (256 bytes RAM + 4 KB ROM) — fast but limited. External memory is off-chip (up to 64 KB RAM + 64 KB ROM) — larger but slower, accessed through P0/P2 using the MOVX/MOVC instructions.

# Experiment 6 — Arithmetic Instruction Programming in 8051

1.  Describe the purpose of studying data transfer and arithmetic operations on external memory.

Ans.  To learn how data is stored in and retrieved from external RAM (using MOVX) and arithmetic performed on it — needed when the on-chip memory is not sufficient.

2.  Define external memory.

Ans.  Memory located outside the chip (up to 64 KB RAM and 64 KB ROM) that is accessed through the address and data buses using the MOVX and MOVC instructions.

# Experiment 7 — Boolean & Logical Instructions Programming (8051)

1.  Explain the purpose of the ANL instruction in 8051 programming.

Ans.  It performs a bitwise logical AND between the destination (A or a byte) and the source, and is mainly used to mask or clear selected bits.

2.  Illustrate how to clear a bit in a byte using the 8051.

Ans.  AND the byte with a mask having 0 at the bit to clear, e.g. ANL A,#0FEH clears bit 0. For a bit-addressable location use CLR directly, e.g. CLR P1.0.

# Experiment 8 — Blinking LED using 8051

1.  What is the 8051 microcontroller?

Ans.  An 8-bit single-chip microcontroller with 4 KB ROM, 128 bytes RAM, four 8-bit I/O ports, two timers, a serial port and an interrupt system.

2.  What is the function of Port 1?

Ans.  Port 1 (P1) is an 8-bit bidirectional general-purpose I/O port.

3.  Which pin of Port 1 is used for the LED?

Ans.  P1.0 — the first pin of Port 1.

4.  Why is a resistor connected in series with the LED?

Ans.  To act as a current-limiting resistor that limits the current and protects the LED from damage (typically 220–330 Ω).

5.  What is the purpose of the crystal oscillator?

Ans.  It provides the clock signal that sets the operating and timing frequency of the microcontroller.

6.  Why is 11.0592 MHz commonly used with the 8051?

Ans.  Because it divides evenly to give exact standard baud rates for serial (UART) communication.

7.  What is the purpose of ORG 0000H?

Ans.  An assembler directive that places the program starting at address 0000H, the reset location of the 8051.

8.  How to create a hex file?

Ans.  Write and compile the program in an IDE such as Keil and build the project — the assembler generates the .hex file.

9.  How many ports does the 8051 have?

Ans.  Four 8-bit I/O ports — P0, P1, P2 and P3 (32 I/O lines in total).

# Experiment 9 — AVR Arithmetic & Logical Operations (Viva)

1.  What is the purpose of the LDI instruction?

Ans.  Load Immediate — it loads an 8-bit constant into a register R16–R31, e.g. LDI R16, 0x05.

2.  Where is the result of the MUL instruction stored?

Ans.  In the register pair R1:R0 — the high byte in R1 and the low byte in R0.

3.  Which AVR instruction performs XOR?

Ans.  The EOR (Exclusive-OR) instruction, e.g. EOR R16, R17.

4.  What does the COM instruction do?

Ans.  It takes the one’s complement of a register — inverts all its bits (R = 0xFF − R).

5.  What is a HEX file?

Ans.  An Intel-HEX file holding the machine code in hexadecimal, which is loaded into the microcontroller’s program (flash) memory.

# Experiment 10 — Read Push-Button Switch & Display on LED (8051)

1.  Which port is used to read the push button?

Ans.  Port 1 — the switch is connected to pin P1.0.

2.  Which pin is used to control the LED?

Ans.  P2.0.

3.  Why is FFH written to Port 1?

Ans.  Writing MOV P1,#0FFH configures Port 1 as an input, so that P1.0 can correctly read the push-button status.

4.  What happens to P1.0 when the switch is pressed?

Ans.  P1.0 is connected to ground and reads logic 0 (LOW); the program then turns the LED at P2.0 ON.

5.  What is the function of the JB instruction?

Ans.  Jump if Bit set — it tests a bit and jumps if that bit is 1, e.g. JB P1.0, NOT_PRESSED.

# Experiment 11 — Interfacing Buzzer with AVR Board

1.  What is the function of a buzzer in an embedded system?

Ans.  It is an output device that produces an audible sound for alarms, alerts or status indication.

2.  Why is PB0 configured as an output in this experiment?

Ans.  Because the buzzer is driven by PB0, which must be an output to send the HIGH/LOW signals that switch the buzzer ON and OFF.

3.  What is the purpose of the DDRB register?

Ans.  Data Direction Register for Port B — it sets each pin as input (0) or output (1); DDRB |= (1<<PB0) makes PB0 an output.

4.  What is the difference between an active buzzer and a passive buzzer?

Ans.  An active buzzer has a built-in oscillator and sounds on a steady DC voltage; a passive buzzer has none and needs an external AC/PWM signal to produce sound.

5.  What is the function of PORTB in this program?

Ans.  It sets the output level of Port B — PORTB |= (1<<PB0) drives PB0 HIGH (buzzer ON) and PORTB &= ~(1<<PB0) drives it LOW (buzzer OFF).

# Experiment 12 — Interfacing 7-Segment Display with AVR Board

1.  What is a 7-segment display?

Ans.  An LED display made of seven segments (a–g) plus a decimal point, which are lit in combinations to show the digits 0–9.

2.  What is the difference between common-cathode and common-anode displays?

Ans.  In a common-cathode display all cathodes join to ground and a segment glows when driven HIGH; in a common-anode display all anodes join to +V and a segment glows when driven LOW.

3.  Why are 330 Ω resistors used with the LED segments?

Ans.  As current-limiting resistors in series with each segment to protect the LEDs from excess current.

4.  Which ATmega32 port is used in this experiment?

Ans.  PORTD — pins PD0–PD7 drive the segments a–g and dp.

5.  What does DDRD = 0xFF do?

Ans.  It configures all eight pins of Port D as outputs.

6.  Why is F_CPU defined as 8 MHz?

Ans.  It tells the compiler the CPU clock is 8 MHz so that _delay_ms() produces accurate delays.

7.  What hexadecimal code is used to display digit 8?

Ans.  0x7F — all seven segments a–g are turned ON.