Experiment 4 — IC Engine Components Design of Mechanisms (BE05000341) · Quiz — Short Answers

Q1.  Valve-gear mechanism — function and layout in vertical and horizontal engines.

Function: opens and closes the inlet and exhaust valves at the correct crank angles so the cylinder breathes properly. In a 4-stroke engine the camshaft turns at ½ crank speed (each valve opens once in every two revolutions).

Main parts: cam & camshaft → tappet (follower) → push rod → rocker arm → valve stem → valve; the valve spring closes the valve and keeps the follower on the cam.

Vertical engine (OHV): camshaft near the crank, valves in the cylinder head → push rod + rocker arm are used so motion travels upward.

Horizontal engine: cylinder lies on its side; the camshaft and valves lie beside the cylinder, so a short tappet (sometimes a rocker) opens the valve almost directly — push rods may not be needed.

Q2.  Numerical — Tubular push rod for an exhaust valve (valve dia = 50 mm, max cyl pressure = 0.4 N/mm², L = 110 mm, E = 2.1 × 10⁵ N/mm², FOS = 4, di/do = 0.75).

Load on push rod Pp = (π/4) · dv² · p = (π/4) · 50² · 0.4 = 785 N ≈ 750 N.
Buckling load required Pcr = Pp × FOS = 3000 N.
Assume both ends hinged, Euler: Pcr = π² E I / L². I = π/64 · (do⁴ − di⁴), with di = 0.75 do → I = 0.0335 do⁴.
3000 = π² × 2.1 × 10⁵ × 0.0335 do⁴ / 110² → do ≈ 6.03 mm → select do = 6.5 mm, di = 0.75 × 6.5 ≈ 4.9 mm. Rechecked FOS ≈ 4.87 (safe).

Ans: Load Pp = 750 N, buckling load Pcr = 3000 N. Push rod: do = 6.5 mm, di ≈ 4.9 mm, L = 110 mm. Actual FOS ≈ 4.87 (> 4, safe).

Q3.  Numerical — Valve-gear design for a horizontal gas engine (port 70 mm, lift 25 mm, p_max = 4.5 MPa; valve opens 33° before ODC, closes 1° after IDC; speed 350 rpm; rocker arms 175 mm each at 140°).

Approach. (1) size the valve by port area and strength, (2) design the valve spring to close the valve and resist suction, (3) design the rocker lever for bending and (4) choose the pins and cam. Key relations used:

• Valve head thickness t = k · d_v √(p/σb); stem diameter d_s = 0.25 d_v + 11 (approx).
• Spring force F_min = p_s · A_v + W_v; F_max = F_min + k·lift.
• Rocker reaction R = P_cam + P_valve; bending at fulcrum M = P × arm; I-section b/d ratio chosen 1 : 2.
• Cam angle for lift (from engine timing) = (θ_open) × (camshaft/crank) = (33° + 180° + 1°)/2 = 107° total; half (rise) = 53.5°.

Valve. d_v ≈ port dia 70 mm → stem d_s = 15 mm, head thickness t = 7 mm, lift = 25 mm.

Spring. Wire d = 4.5 mm, mean coil D = 45 mm, 10 active (12 total) coils, free length 118 mm, forces 140 N (closed) to 254 N (lift).

Rocker lever. Design load P ≈ 1202 N; fulcrum reaction R ≈ 2259 N; I-section at fulcrum H = 35 mm, B = 28 mm, flange t = 7 mm; each arm 175 mm.

Pins. Fulcrum pin 14 mm φ × 18 mm; roller pin 10 mm φ × 13 mm.

Cam. Rise/return each 53.5°, lift 25 mm, base-circle radius 35 mm (base circle 70 mm with a 30 mm roller) → prime-circle radius ≈ 85 mm.

Ans: Valve: stem 15 mm, head 7 mm, lift 25 mm. Spring: wire 4.5 mm, mean coil 45 mm, 10 active coils, free length 118 mm, force 140 → 254 N. Rocker: load 1202 N, fulcrum reaction 2259 N, I-section H = 35, B = 28, t = 7 mm, arms 175 mm. Pins: fulcrum 14 × 18 mm, roller 10 × 13 mm. Cam: rise/return 53.5°, lift 25 mm, prime-circle radius ≈ 85 mm.