ASSIGNMENT  No. 2

Microprocessors and Microcontrollers

8085  ·  8051  ·  AVR     —     Complete Solution

| Particulars | Details |
| --- | --- |
| Name of the student |  |
| Enrolment / Roll number |  |
| Class / Division |  |
| Subject | Microprocessors and Microcontrollers |
| Assignment number | Assignment No. 2 |
| Number of questions attempted | All 10 questions |
| Date of submission |  |

CONTENTS

| Q. No. | Question |
| --- | --- |
| Q.1 | Explain the following instructions of the 8085 microprocessor with examples — RLC, STC, CPI, RET, PUSH, XRA, XCHG, RAL, CMP, LXI, CMA, POP, JMP, CALL |
| Q.2 | Explain the following 8051 instructions with an example — (i) MOV (ii) ANL C (iii) CJNE (iv) DIV (v) MUL |
| Q.3 | Explain PC and DPTR with their significance. Write a program to interchange the contents of PC and DPTR |
| Q.4 | Write an assembly-level program for the 8051 to subtract 156F8H from 24759CH and save the result in RAM locations starting at 50H |
| Q.5 | Draw and explain the 8085 architecture |
| Q.6 | Describe the status register of the AVR |
| Q.7 | Write a program to transfer the contents of R0 of bank 0 to register R7 of bank 1 |
| Q.8 | Write a brief note on the addressing modes of the AVR microcontroller |
| Q.9 | Explain in brief the hardware modules of the AVR with a suitable diagram |
| Q.10 | Compare AJMP, LJMP and SJMP in the microcontroller |

# Q.1   Explain the following instructions of the 8085 microprocessor with examples

The 8085 instruction set contains 74 instruction types (246 opcodes) which are classified into five groups — data-transfer, arithmetic, logical, branching and machine-control. For each instruction asked below, the opcode, the number of bytes, the addressing mode, the flags affected, the operation performed and a worked example are given.

In the signal and instruction names below, a bar over a name (for example INTA, RD, WR) denotes an active-low signal; such names are written without the bar in running text.

## Summary of the fourteen instructions

| # | Mnemonic | Opcode (hex) | Bytes | Group | Flags affected |
| --- | --- | --- | --- | --- | --- |
| 1 | RLC | 07 | 1 | Logical (rotate) | CY only |
| 2 | STC | 37 | 1 | Logical | CY only (set to 1) |
| 3 | CPI data | FE | 2 | Arithmetic | All five flags |
| 4 | RET | C9 | 1 | Branching | None |
| 5 | PUSH rp | C5 / D5 / E5 / F5 | 1 | Data transfer | None |
| 6 | XRA r | A8 – AF | 1 | Logical | S, Z, P; CY = 0, AC = 0 |
| 7 | XCHG | EB | 1 | Data transfer | None |
| 8 | RAL | 17 | 1 | Logical (rotate) | CY only |
| 9 | CMP r | B8 – BF | 1 | Arithmetic | All five flags |
| 10 | LXI rp, data16 | 01 / 11 / 21 / 31 | 3 | Data transfer | None |
| 11 | CMA | 2F | 1 | Logical | None |
| 12 | POP rp | C1 / D1 / E1 / F1 | 1 | Data transfer | Only POP PSW (restores them) |
| 13 | JMP addr16 | C3 | 3 | Branching | None |
| 14 | CALL addr16 | CD | 3 | Branching | None |

### 1)   RLC  —  Rotate Accumulator Left

Opcode 07H  ·  1 byte  ·  Addressing mode: implied  ·  Flags: CY only

Every bit of the accumulator is shifted one position to the left. The bit shifted out of the most-significant position (D7) is placed both in the least-significant position (D0) and in the carry flag. It is therefore a true 8-bit circular rotation — no data is lost, and the accumulator returns to its original value after eight RLC instructions.

MVI A, 97H ; A = 1001 0111 = 97H RLC ; rotate left, circular Before : CY = x A = 1 0 0 1 0 1 1 1 (97H) After : CY = 1 A = 0 0 1 0 1 1 1 1 (2FH) D7 (=1) has gone into both D0 and CY.

Use: RLC multiplies an unsigned number by 2 (when the MSB is 0) and is the standard way of testing the bits of a byte one after another, because each rotation brings a fresh bit into the carry which can then be tested with JC / JNC.

### 2)   STC  —  Set Carry

Opcode 37H  ·  1 byte  ·  Addressing mode: implied  ·  Flags: CY is set to 1; no other flag changes

STC unconditionally forces the carry flag to logic 1. The accumulator, the registers and the remaining four flags are left completely undisturbed. The companion instruction is CMC (3FH), which complements the carry.

MVI A, 00H ; A = 00H, CY is unknown STC ; CY = 1 CMC ; CY = 0 (complement of 1) A still contains 00H and S, Z, P, AC are unchanged.

Use: to set up a known borrow before a subtract-with-borrow sequence, to return a one-bit TRUE/FALSE status from a subroutine in the carry flag, and (with CMC) to clear the carry in two bytes without disturbing the accumulator.

### 3)   CPI  8-bit data  —  Compare Immediate with Accumulator

Opcode FEH  ·  2 bytes  ·  Addressing mode: immediate  ·  Flags: all five

The 8-bit immediate data is subtracted from the accumulator internally, but the result is thrown away — neither the accumulator nor the operand is changed. Only the flags record the outcome, so CPI followed by a conditional jump gives a complete comparison.

| Condition | CY | Z | Conditional jump that is taken |
| --- | --- | --- | --- |
| A < data | 1 | 0 | JC (jump on carry) |
| A = data | 0 | 1 | JZ (jump on zero) |
| A > data | 0 | 0 | JNC and JNZ are both true |

MVI A, 64H ; A = 64H = 100 decimal CPI 50H ; 64H - 50H = 14H ; result is non-zero and positive ; => Z = 0, CY = 0, A is STILL 64H JNC BIG ; this jump IS taken, because A > 50H ... BIG: ...

Use: limit checking, searching a data block for a particular byte, and validating keyboard/ASCII input ranges.

### 4)   RET  —  Return from Subroutine (unconditional)

Opcode C9H  ·  1 byte  ·  Addressing mode: register indirect (through SP)  ·  Flags: none

RET takes the two bytes lying on the top of the stack and loads them into the program counter, so execution continues from the instruction that follows the CALL which invoked the subroutine.

Operation : PCL <- M[SP] PCH <- M[SP + 1] SP <- SP + 2

LXI SP, 27FFH ; the stack MUST be initialised first CALL DELAY ; return address (of MOV A,B) is pushed MOV A, B ; <-- control comes back exactly here HLT DELAY: MVI C, 0FFH ; subroutine LOOP: DCR C JNZ LOOP RET ; PC <- address of MOV A, B

Key point — every CALL must be balanced by exactly one RET, and the number of PUSH instructions inside the subroutine must equal the number of POP instructions. If the stack is unbalanced when RET executes, the processor returns to a wrong address and the program crashes.

The 8085 also provides eight conditional returns — RZ, RNZ, RC, RNC, RP, RM, RPE and RPO — which return only if the tested flag condition is true.

### 5)   PUSH rp  —  Push Register Pair on the Stack

Opcodes PUSH B = C5H, PUSH D = D5H, PUSH H = E5H, PUSH PSW = F5H  ·  1 byte  ·  Addressing mode: register indirect  ·  Flags: none

The 16-bit contents of the specified register pair are copied on to the stack. The stack pointer is decremented before each byte is written, so the 8085 stack grows towards lower memory addresses.

Operation : SP <- SP - 1 M[SP] <- higher-order register (B, D, H or A) SP <- SP - 1 M[SP] <- lower-order register (C, E, L or flags)

LXI SP, 2000H LXI B, 1234H ; B = 12H, C = 34H PUSH B After PUSH B : memory 1FFFH = 12H (contents of B) memory 1FFEH = 34H (contents of C) SP = 1FFEH

PUSH PSW saves the Program Status Word — the accumulator goes to the higher location and the flag register to the lower one. It is the only way to preserve the flags across a subroutine call.

### 6)   XRA r / XRA M  —  Exclusive-OR with the Accumulator

Opcodes A8H–AFH (XRA M = AEH)  ·  1 byte  ·  Addressing mode: register / register indirect  ·  Flags: S, Z and P from the result; CY and AC are always reset to 0

A bit-by-bit exclusive-OR is performed between the accumulator and the operand, and the result is placed in the accumulator:  A ← A ⊕ operand.

MVI A, 54H ; A = 0101 0100 MVI B, 78H ; B = 0111 1000 XRA B ; exclusive-OR bit by bit A = 0 1 0 1 0 1 0 0 (54H) B = 0 1 1 1 1 0 0 0 (78H) ------------------------ XOR A = 0 0 1 0 1 1 0 0 (2CH) CY = 0, AC = 0

Special uses of XRA

- XRA A is the standard idiom to clear the accumulator and the carry together in a single byte, and it also sets Z = 1 (shorter and faster than MVI A, 00H followed by clearing CY).

- Selective bit toggling — a 1 in the mask inverts the corresponding bit of A, a 0 leaves it unchanged.

- Equality test — if A ⊕ operand gives zero (Z = 1) the two bytes were identical.

### 7)   XCHG  —  Exchange H-L with D-E

Opcode EBH  ·  1 byte  ·  Addressing mode: register  ·  Flags: none

The contents of register H are exchanged with D, and simultaneously L is exchanged with E. It is a true exchange — no intermediate register is needed and nothing is lost.

LXI H, 2050H ; H = 20H, L = 50H LXI D, 30A0H ; D = 30H, E = A0H XCHG Before : HL = 2050H DE = 30A0H After : HL = 30A0H DE = 2050H

Use: HL is the only register pair that can be used as a memory pointer (the operand M in instructions such as MOV A, M or ADD M). In block-transfer and string routines two pointers are needed; the second one is kept in DE and brought into HL whenever required by a single XCHG.

### 8)   RAL  —  Rotate Accumulator Left through Carry

Opcode 17H  ·  1 byte  ·  Addressing mode: implied  ·  Flags: CY only

RAL is a nine-bit rotation in which the carry flag takes part as the ninth bit. The MSB (D7) moves into the carry, and the previous value of the carry moves into the LSB (D0).

STC ; make CY = 0 ... CMC ; ... by set-then-complement MVI A, 97H ; A = 1001 0111 RAL Before : CY = 0 A = 1 0 0 1 0 1 1 1 (97H) After : CY = 1 A = 0 0 1 0 1 1 1 0 (2EH) D7 (=1) went into CY; the old CY (=0) came into D0.

### Difference between RLC and RAL

| Point | RLC (07H) | RAL (17H) |
| --- | --- | --- |
| Length of the rotation | 8-bit circular rotation | 9-bit rotation (carry included) |
| Bit that enters D0 | The old D7 itself | The old value of the carry flag |
| Bit that enters CY | The old D7 | The old D7 |
| Is the carry part of the data path? | No — the carry only receives a copy of D7 | Yes — the carry is a genuine ninth bit |
| Typical application | Testing the bits of a single byte; multiplying a byte by 2 | Multi-byte shifts and multiplication, where the carry carries a bit from one byte into the next |

The corresponding right-rotate instructions are RRC (0FH, 8-bit circular) and RAR (1FH, 9-bit through carry).

### 9)   CMP r / CMP M  —  Compare Register or Memory with the Accumulator

Opcodes B8H–BFH (CMP M = BEH)  ·  1 byte  ·  Addressing mode: register / register indirect  ·  Flags: all five

The specified operand is subtracted from the accumulator internally. Neither operand is modified; only the flags are set, exactly as for CPI (CY = 1 if A is smaller, Z = 1 if they are equal).

MVI A, 50H MVI B, 50H CMP B ; 50H - 50H = 00H ; => Z = 1, CY = 0, A is still 50H JZ EQUAL ; this jump is taken Finding the larger of two bytes held in B and C : MOV A, B CMP C ; compare B with C JNC BIGGER ; CY = 0 => B >= C MOV A, C ; otherwise C is larger BIGGER: STA 3000H ; store the larger number

CMP is used for sorting, for finding the largest or smallest element of an array, and for all loop-termination tests that compare two variable quantities (CPI is used when one of them is a constant).

### 10)   LXI rp, 16-bit data  —  Load Register Pair Immediate

Opcodes LXI B = 01H, LXI D = 11H, LXI H = 21H, LXI SP = 31H  ·  3 bytes  ·  Addressing mode: immediate  ·  Flags: none

A 16-bit constant written in the instruction itself is loaded into the specified register pair. The lower byte goes into the lower-order register and the higher byte into the higher-order register. In memory the instruction is stored opcode-first, then the low byte, then the high byte (little-endian).

LXI H, 2050H ; H = 20H, L = 50H -> HL points to 2050H LXI D, 3000H ; D = 30H, E = 00H LXI B, 000AH ; B = 00H, C = 0AH (counter = 10) LXI SP, 27FFH ; initialise the stack pointer "LXI H, 2050H" occupies three memory locations : 2000H : 21H <- opcode 2001H : 50H <- lower byte -> goes to L 2002H : 20H <- higher byte -> goes to H

Use: LXI is the normal way to set up a memory pointer in HL, to load a 16-bit counter, and above all to initialise the stack pointer — which must be done before any PUSH, CALL or interrupt can be used.

### 11)   CMA  —  Complement Accumulator

Opcode 2FH  ·  1 byte  ·  Addressing mode: implied  ·  Flags: none at all

Each bit of the accumulator is inverted, which gives the 1’s complement of the number. The fact that CMA affects no flag — not even the carry — is a point frequently missed.

MVI A, 89H CMA Before : A = 1 0 0 0 1 0 0 1 (89H) After : A = 0 1 1 1 0 1 1 0 (76H) no flag changes 2's complement (negation) of A : CMA ; 1's complement INR A ; + 1 => 2's complement

Use: forming the 1’s and 2’s complements for signed arithmetic and for subtraction by addition, and inverting a bit mask before an ANA instruction (to clear selected bits).

### 12)   POP rp  —  Pop the Stack into a Register Pair

Opcodes POP B = C1H, POP D = D1H, POP H = E1H, POP PSW = F1H  ·  1 byte  ·  Addressing mode: register indirect  ·  Flags: none, except POP PSW which restores all five

Two bytes are removed from the top of the stack and placed in the specified register pair. The stack pointer is incremented after each byte is read, so the stack shrinks towards higher addresses — exactly the reverse of PUSH.

Operation : lower-order register <- M[SP] SP <- SP + 1 higher-order register <- M[SP] SP <- SP + 1

Given SP = 1FFEH, M[1FFEH] = 34H, M[1FFFH] = 12H POP B After POP B : C = 34H, B = 12H, SP = 2000H

Key point — the stack is a LIFO (last-in, first-out) structure, so the POP instructions inside a subroutine must appear in the reverse order of the PUSH instructions. PUSH B / PUSH D must be undone by POP D / POP B; writing POP B / POP D silently swaps the contents of the two pairs.

### 13)   JMP 16-bit address  —  Unconditional Jump

Opcode C3H  ·  3 bytes  ·  Addressing mode: immediate  ·  Flags: none

The 16-bit address given in the instruction is loaded into the program counter (PC ← address), so the processor continues execution from that location instead of the next sequential one. No return address is saved, therefore control cannot come back automatically.

LXI H, 2000H ; initialise pointer MVI C, 05H ; counter = 5 BACK: MOV A, M ; read a byte INX H ; next location DCR C JNZ BACK ; conditional jump (loop 5 times) JMP FINISH ; unconditional jump - always taken ... FINISH: HLT

Besides JMP, the 8085 has eight conditional jumps — JZ, JNZ, JC, JNC, JP, JM, JPE and JPO — each of which is also 3 bytes long and tests one flag.

### 14)   CALL 16-bit address  —  Unconditional Subroutine Call

Opcode CDH  ·  3 bytes  ·  Addressing mode: immediate (with register-indirect stack access)  ·  Flags: none

CALL first pushes the address of the next instruction (the contents of the PC after the 3-byte CALL has been fetched) on to the stack, and then jumps to the given address. Because the return address is preserved, a RET at the end of the subroutine can bring control back.

Operation : SP <- SP - 1 M[SP] <- PCH ; higher byte of return address SP <- SP - 1 M[SP] <- PCL ; lower byte of return address PC <- 16-bit address given in the instruction

LXI SP, 27FFH ; stack must be initialised LXI H, 2000H CALL SUM ; 2003H (say) is pushed on the stack STA 3000H ; <-- RET brings control back here HLT SUM: MOV A, M ; subroutine : add two bytes INX H ADD M RET ; PC is reloaded from the stack

### Difference between JMP and CALL

| Point | JMP addr16 | CALL addr16 |
| --- | --- | --- |
| Return address | Not saved | Pushed on to the stack |
| Effect on the stack pointer | None | SP is decremented by 2 |
| Can control come back? | No | Yes, by means of RET |
| Machine cycles / T-states | 3 cycles, 10 T-states | 5 cycles, 18 T-states |
| Purpose | Loops and unconditional transfer of control | Calling a reusable subroutine |

Seven conditional calls (CZ, CNZ, CC, CNC, CP, CM, CPE, CPO) are also available, and the eight one-byte RST n instructions perform a call to the fixed vector address n × 8.

# Q.2   Explain the following 8051 instructions with an example

The 8051 has a 255-instruction opcode set built around the accumulator A, the register banks R0–R7, the bit-addressable carry flag and the 16-bit data pointer. The five instructions asked for are explained below.

## (i)   MOV  —  Move (copy) Data

MOV copies a byte (or in one special case a 16-bit constant) from a source to a destination inside the on-chip memory space — the accumulator, the working registers, internal RAM and the special function registers. The source is never altered, so MOV is a copy and not a transfer. MOV does not affect any flag, unless the PSW itself is the destination.

Important distinction — MOV can only reach internal memory. External RAM is accessed with MOVX (MOVX A, @DPTR) and the program/code memory with MOVC (MOVC A, @A+DPTR). Writing MOV A, @DPTR is an error.

Permitted forms of MOV

| Form | Addressing mode | Example | Result |
| --- | --- | --- | --- |
| MOV A, #data | Immediate | MOV A, #25H | A = 25H |
| MOV A, Rn | Register | MOV A, R3 | A = contents of R3 |
| MOV A, direct | Direct | MOV A, 40H | A = contents of RAM location 40H |
| MOV A, @Ri | Register indirect | MOV A, @R0 | A = contents of the location R0 points to |
| MOV Rn, #data | Immediate | MOV R5, #0FFH | R5 = FFH |
| MOV direct, A | Direct | MOV 30H, A | RAM 30H = A |
| MOV direct, direct | Direct | MOV 30H, 40H | RAM 30H = contents of RAM 40H |
| MOV @Ri, A | Register indirect | MOV @R1, A | The location R1 points to = A |
| MOV DPTR, #data16 | Immediate (16-bit) | MOV DPTR, #2500H | DPH = 25H, DPL = 00H |
| MOV C, bit | Bit (direct bit) | MOV C, P1.3 | Carry flag = pin P1.3 |
| MOV bit, C | Bit (direct bit) | MOV P2.0, C | Pin P2.0 = carry flag |

Example program

ORG 0000H MOV A, #35H ; immediate : A = 35H MOV R0, #40H ; R0 is used as a pointer to RAM 40H MOV @R0, A ; indirect : RAM 40H = 35H MOV R2, A ; register : R2 = 35H MOV 30H, #0FFH ; direct : RAM 30H = FFH MOV A, 30H ; direct : A = FFH MOV DPTR, #2500H ; the only 16-bit MOV : DPH=25H, DPL=00H MOV P1, #0AAH ; SFR as destination : port 1 = 1010 1010 SJMP $ END

## (ii)   ANL C, bit  —  Bit-wise Logical AND with the Carry Flag

This is the bit-level form of the ANL instruction. The carry flag acts as the Boolean accumulator of the 8051: it supplies one input of the AND gate, receives the result, and the addressed bit is left unchanged.

| Instruction | Bytes | Operation | Flags |
| --- | --- | --- | --- |
| ANL C, bit | 2 | CY ← CY AND (bit). The addressed bit is not modified. | CY only |
| ANL C, /bit | 2 | CY ← CY AND (NOT bit). The bit itself is still not modified — only its complement is used. | CY only |

Any bit that has a direct bit address may be used as the operand: the bits of the bit-addressable RAM (20H–2FH, i.e. bit addresses 00H–7FH), the port pins (P0–P3), and the bit-addressable SFRs such as PSW, ACC, TCON and IE.

Example 1 — realising a two-input AND gate in software. Inputs on P1.0 and P1.1, output on P1.2, i.e. P1.2 = P1.0 · P1.1

MOV C, P1.0 ; C <- first input ANL C, P1.1 ; C <- P1.0 AND P1.1 MOV P1.2, C ; send the result out on P1.2

Example 2 — using the complement form. Realise Y = A · B where A = P1.0 and B = P1.1, output on P1.2

MOV C, P1.0 ; C <- A ANL C, /P1.1 ; C <- A AND (NOT B) - P1.1 is NOT changed MOV P1.2, C ; Y = A . B(bar)

Example 3 — a three-input gate, Y = X1 · X2 · X3 with the inputs taken from bit-addressable RAM:

MOV C, 20H.0 ; C <- X1 ANL C, 20H.1 ; C <- X1 . X2 ANL C, 20H.2 ; C <- X1 . X2 . X3 MOV P2.7, C ; output

This bit-processing ability (together with ORL C, CPL C, SETB, CLR, MOV C) is why the 8051 is so well suited to replacing small amounts of random logic and to writing relay-ladder style control programs.

## (iii)   CJNE  —  Compare and Jump if Not Equal

3 bytes  ·  2 machine cycles  ·  Flags: CY only

CJNE compares the destination operand with the source operand and jumps to the given relative address if they are not equal; if they are equal, execution simply falls through to the next instruction. Neither operand is modified. In addition the carry flag is set when destination < source, so CJNE followed by JC or JNC yields a complete three-way (less-than / equal / greater-than) decision.

| Form | Comparison performed |
| --- | --- |
| CJNE A, #data, rel | Accumulator with an immediate constant |
| CJNE A, direct, rel | Accumulator with the contents of a direct address |
| CJNE Rn, #data, rel | Register R0–R7 with an immediate constant |
| CJNE @Ri, #data, rel | The byte pointed to by R0 or R1 with an immediate constant |

Example 1 — wait until port P1 reads 55H

WAIT: MOV A, P1 CJNE A, #55H, WAIT ; keep looping while P1 is NOT 55H ; control reaches here only when P1 = 55H MOV P2, #0FFH

Example 2 — a complete three-way test of the value in R0 against 60H

MOV A, R0 CJNE A, #60H, NOTEQ ; not equal -> jump SJMP EQUAL ; falls through : A = 60H NOTEQ: JC LOWER ; CY = 1 => A < 60H SJMP HIGHER ; CY = 0 => A > 60H EQUAL: MOV P1, #01H SJMP DONE LOWER: MOV P1, #02H SJMP DONE HIGHER: MOV P1, #04H DONE: SJMP $

Example 3 — using CJNE as a loop counter (counting down from 10 to 1)

MOV R2, #10 AGAIN: MOV P1, R2 DJNZ R2, AGAIN ; or: DEC R2 / CJNE R2, #0, AGAIN

Note — the jump is relative, so the target must lie within −128 to +127 bytes of the instruction that follows CJNE. The 8051 has no "compare and jump if equal" instruction; equality is handled by letting the program fall through, as in Example 1.

## (iv)   DIV AB  —  Unsigned 8-bit Division

Opcode 84H  ·  1 byte  ·  4 machine cycles  ·  Flags: CY is always cleared; OV signals division by zero

DIV AB divides the unsigned 8-bit number in the accumulator by the unsigned 8-bit number in register B. The quotient is left in A and the remainder in B. The operands are fixed — only A and B can be used.

Operation : A <- quotient of A / B B <- remainder of A / B CY <- 0 (always) OV <- 1 if B = 0 (result undefined), else 0

Example — divide 95 by 10

MOV A, #95 ; A = 5FH = 95 decimal (dividend) MOV B, #10 ; B = 0AH = 10 decimal (divisor) DIV AB ; A = 09 (quotient) ; B = 05 (remainder) ; CY = 0, OV = 0 MOV R0, A ; save the quotient MOV R1, B ; save the remainder

Example — checking for division by zero

MOV A, R5 MOV B, R6 DIV AB JB OV, ERROR ; OV = 1 means R6 was 0 -> handle the error MOV 40H, A ; otherwise store the quotient SJMP DONE ERROR: MOV P1, #0FFH ; signal "divide by zero" DONE: SJMP $

Use: the most common application is binary-to-decimal (BCD/ASCII) conversion, where a byte is divided repeatedly by 10 to extract its decimal digits, and scaling of ADC readings.

## (v)   MUL AB  —  Unsigned 8-bit Multiplication

Opcode A4H  ·  1 byte  ·  4 machine cycles  ·  Flags: CY is always cleared; OV indicates a result larger than 255

MUL AB multiplies the unsigned byte in the accumulator by the unsigned byte in register B. The product is 16 bits wide: the low byte is returned in A and the high byte in B.

Operation : A <- lower byte of A x B B <- higher byte of A x B CY <- 0 (always) OV <- 1 if the product > FFH (i.e. if B is non-zero)

Example — multiply 37 by 101

MOV A, #25H ; A = 25H = 37 decimal MOV B, #65H ; B = 65H = 101 decimal MUL AB ; 37 x 101 = 3737 = 0E99H ; A = 99H (lower byte) ; B = 0EH (higher byte) ; OV = 1 (product > 255), CY = 0 MOV 50H, A ; store the low byte MOV 51H, B ; store the high byte

Example — a product that fits in one byte

MOV A, #0AH ; 10 MOV B, #05H ; 5 MUL AB ; 10 x 5 = 50 = 32H ; A = 32H, B = 00H, OV = 0 JNB OV, SMALL ; OV = 0 -> the whole answer is in A

### Common features of MUL AB and DIV AB

- Both work only on the fixed pair of registers A and B — no other register may be used.

- Both handle unsigned numbers only; signed multiplication and division must be written in software.

- Both always clear the carry flag and use OV to report the special condition (overflow for MUL, division by zero for DIV).

- Both take 4 machine cycles (48 clock periods in the standard 8051), which makes them the slowest instructions in the instruction set — every other 8051 instruction takes 1 or 2 machine cycles.

- Both are 1-byte instructions, so they are very compact in terms of code memory.

# Q.3   Explain PC and DPTR with their significance. Write a program to interchange the contents of PC and DPTR

## A.   The Program Counter (PC)

The program counter is the 16-bit register that holds the address of the next byte of code to be fetched from the program (code) memory. It is the register that makes the processor execute a program at all: after every byte is fetched the PC is automatically incremented, so the processor walks through the program in sequence.

- Width — 16 bits. This is what fixes the size of the code space at 2^16 = 64 KB, addresses 0000H to FFFFH.

- It is not a special function register. The PC has no direct address in the SFR space, therefore it cannot be read or written by a MOV instruction. This is a deliberate design decision — it prevents a program from corrupting its own flow of control by accident.

- Only the program-flow instructions change it: SJMP, AJMP, LJMP, ACALL, LCALL, RET, RETI, JMP @A+DPTR, all the conditional jumps (JZ, JNZ, JC, JNC, JB, JNB, CJNE, DJNZ …), a hardware interrupt, and RESET.

- Reset value = 0000H. On power-up or reset the PC is cleared, so the very first instruction of every 8051 program is fetched from address 0000H. Because the first interrupt vector is only three bytes further on (at 0003H), the instruction placed at 0000H is normally the 3-byte LJMP MAIN.

- It is saved on the stack automatically. When ACALL/LCALL is executed, or when an interrupt is accepted, the hardware pushes the PC — low byte first, then high byte — and RET/RETI pops it back.

Significance — the PC is the "book-mark" of the CPU. It enforces sequential execution, it is the sole mechanism by which branching and subroutine calls work, and the entire purpose of the stack during a call or an interrupt is to save and later restore it. Without the PC the processor would have no notion of "where it is" in the program.

## B.   The Data Pointer (DPTR)

DPTR is the 16-bit data pointer, and it is the only general-purpose 16-bit register that the 8051 programmer is given. Physically it is two 8-bit special function registers:

| Part | Name | SFR address | Holds |
| --- | --- | --- | --- |
| Higher byte | DPH | 83H | Bits A15 – A8 of the pointer |
| Lower byte | DPL | 82H | Bits A7 – A0 of the pointer |

Because DPH and DPL lie in the SFR space they are directly addressable, so DPTR may be handled either as one 16-bit quantity or as two independent bytes:

MOV DPTR, #2500H ; load all 16 bits at once (DPH=25H, DPL=00H) INC DPTR ; increment the full 16-bit pointer MOV A, DPL ; read the lower half on its own MOV DPH, #30H ; write the higher half on its own MOV A, #00H MOV DPL, A ; DPTR = 3000H, built one byte at a time

The three jobs DPTR performs

| Purpose | Instruction | Explanation |
| --- | --- | --- |
| Accessing external data memory | MOVX A, @DPTR MOVX @DPTR, A | DPTR supplies the full 16-bit address, so the whole 64 KB of external RAM can be reached. |
| Reading look-up tables in code memory | MOVC A, @A+DPTR | DPTR holds the base address of the table and A the index; the byte fetched from Flash/ROM is returned in A. Used for messages, 7-segment codes, sine tables. |
| Computed (indexed) jumps | JMP @A+DPTR | DPTR holds the base of a jump table and A the offset — this is how a jump table or a state machine is implemented. |

Significance — DPTR is the only 16-bit pointer available to the programmer. R0 and R1 can also be used indirectly (MOV A, @R0), but they are 8 bits wide and can therefore address only the 256 bytes of internal RAM. Without DPTR, external memory and look-up tables larger than 256 bytes could not be reached at all.

## C.   Comparison of PC and DPTR

| Point of comparison | Program Counter (PC) | Data Pointer (DPTR) |
| --- | --- | --- |
| Width | 16 bits | 16 bits (DPH : DPL) |
| Present in the SFR space? | No — it has no direct address | Yes — DPH = 83H, DPL = 82H |
| Can a MOV read or write it? | No | Yes — MOV DPTR, #data16, MOV A, DPL, etc. |
| Can it be accessed one byte at a time? | No | Yes, as DPH and DPL |
| What it points to | The next instruction byte in program (code) memory | Data — external RAM, or a constant table in code memory |
| Updated automatically? | Yes — incremented after every byte fetched | No — only by the program (INC DPTR, MOV DPTR, #…) |
| Value after RESET | 0000H | 0000H |
| Changed by | Jumps, calls, returns, interrupts, RESET | MOV, INC DPTR (and nothing else) |
| Saved on the stack by hardware? | Yes, during ACALL / LCALL / interrupt | No — the programmer must push DPH and DPL if required |
| Purpose | Controls the flow of the program | Provides a 16-bit address for data |

## D.   Program to interchange the contents of PC and DPTR

The essential difficulty — the PC is not in the SFR space, so no MOV instruction can read it or write it, and the 8051 has no 16-bit exchange instruction. The exchange must therefore be carried out through the stack, using the only two mechanisms that move data between the PC and the stack:

- To read the PC — execute an LCALL. The hardware pushes the current PC (that is, the address of the instruction following the LCALL), low byte first and then high byte. Two POP instructions then recover it, high byte first.

- To write the PC — PUSH the required 16-bit value on to the stack (low byte first, then high byte) and execute RET. RET pops the high byte first and then the low byte into the PC, so the processor jumps to the value that was pushed.

Program

;-------------------------------------------------------------------- ; Interchange the contents of the PC and the DPTR (8051) ;-------------------------------------------------------------------- ORG 0000H LJMP MAIN ; reset vector ORG 0030H ; start above the interrupt vectors MAIN: MOV SP, #4FH ; initialise the stack in RAM MOV DPTR, #0200H ; the value that must go into the PC ;---- Step 1 : save the present DPTR in R6 (high) and R7 (low) ------ MOV R7, DPL ; R7 = lower byte of DPTR = 00H MOV R6, DPH ; R6 = higher byte of DPTR = 02H ;---- Step 2 : copy the PC into the DPTR ---------------------------- LCALL HERE ; hardware pushes PCL then PCH HERE: POP DPH ; top of stack = PCH -> DPH POP DPL ; next byte = PCL -> DPL ; DPTR now holds the old PC ; (the address of the label HERE) ;---- Step 3 : copy the saved DPTR value into the PC ---------------- PUSH 07H ; R7 = low byte, pushed FIRST PUSH 06H ; R6 = high byte, pushed SECOND RET ; PC <- 0200H ; the interchange is now complete ORG 0200H ; execution continues from here NEW: MOV P1, #0AAH ; (code at the new PC value) SJMP $ END

Explanation of the important points

- PUSH and POP accept only direct addresses, never register names. R6 and R7 of bank 0 occupy the direct addresses 06H and 07H, which is why the instructions are written as PUSH 07H and PUSH 06H.

- The order matters. LCALL pushes PCL first and PCH second, so PCH is on top and must be popped first — hence POP DPH before POP DPL. RET pops PCH first, so the high byte must be pushed last — hence PUSH 07H (low) before PUSH 06H (high). Reversing either pair would exchange the two halves of the address.

- The PC value captured in DPTR is the address of the instruction immediately after the LCALL — i.e. the address of the label HERE — because that is exactly what the processor pushes as the return address.

- Valid code must exist at the new PC value (0200H here), otherwise the processor will begin executing whatever happens to be in the Flash at that address.

- The stack must be initialised and must have room for the pushes; MOV SP, #4FH places it safely above the register banks and the bit-addressable area.

Conclusion — a genuinely simultaneous exchange of PC and DPTR is not possible on the 8051, because the PC cannot be an operand of any data-transfer instruction. What the program above performs is a *read of the PC into DPTR followed by a write of the old DPTR into the PC*, carried out through the stack, which is the accepted way of answering this question.

# Q.4   Write an assembly-level program for the 8051 to subtract 156F8H from 24759CH and save the result in RAM locations starting at 50H

## Step 1 — Analysis of the problem

- Minuend     =  24759CH  →  three bytes :  24H  75H  9CH

- Subtrahend  =  0156F8H  →  three bytes :  01H  56H  F8H  (156F8H written as a full three-byte number)

- The 8051 ALU is only 8 bits wide, so the subtraction must be performed one byte at a time, starting from the least significant byte, with the borrow propagating from each byte to the next.

## Step 2 — Hand calculation

24 75 9C <-- minuend (24759CH) - 01 56 F8 <-- subtrahend (0156F8H) --------------- 23 1E A4 <-- result (231EA4H) Byte by byte : LSB : 9C - F8 -> a borrow is needed 19CH - F8H = A4H , borrow (CY) = 1 middle : 75 - 56 - 1 (borrow) = 1EH , borrow (CY) = 0 MSB : 24 - 01 - 0 (borrow) = 23H , borrow (CY) = 0 Check (decimal) : 2,389,404 - 87,800 = 2,301,604 = 231EA4H OK

## Step 3 — The program

;-------------------------------------------------------------------- ; 8051 program : 24759CH - 0156F8H = 231EA4H ; ; The result is stored in internal RAM, lowest byte first : ; 50H = A4H (least significant byte) ; 51H = 1EH (middle byte) ; 52H = 23H (most significant byte) ;-------------------------------------------------------------------- ORG 0000H CLR C ; no borrow before the first subtraction ;---------------- least significant byte ---------------------------- MOV A, #9CH ; A = LSB of the minuend SUBB A, #0F8H ; A = 9CH - F8H - 0 = A4H , CY = 1 (borrow) MOV 50H, A ; RAM 50H = A4H ;---------------- middle byte --------------------------------------- MOV A, #75H ; A = middle byte of the minuend SUBB A, #56H ; A = 75H - 56H - 1 = 1EH , CY = 0 MOV 51H, A ; RAM 51H = 1EH ;---------------- most significant byte ----------------------------- MOV A, #24H ; A = MSB of the minuend SUBB A, #01H ; A = 24H - 01H - 0 = 23H , CY = 0 MOV 52H, A ; RAM 52H = 23H SJMP $ ; stop here (the 8051 has no HLT) END

## Step 4 — Result

| Internal RAM address | Contents after execution | Significance |
| --- | --- | --- |
| 50H | A4H | Least significant byte of the result |
| 51H | 1EH | Middle byte of the result |
| 52H | 23H | Most significant byte of the result |

Reading the three locations from the highest address downwards gives 23 1E A4 H, which is the required difference.

## Step 5 — Points to note

- The 8051 has no plain SUB instruction. SUBB always performs  A ← A − operand − CY, i.e. it *always* subtracts the borrow as well. Therefore CLR C must be executed before the least-significant byte, otherwise a stale carry from an earlier operation would corrupt the answer by 1.

- CLR C must not be repeated between the bytes — the borrow produced while subtracting one byte is exactly what the next byte needs. This is the whole mechanism of multi-byte subtraction.

- A hexadecimal constant that begins with a letter must be written with a leading zero — #0F8H, not #F8H — otherwise the assembler treats it as a symbol name and reports an error.

- The result is stored little-endian (lowest byte at the lowest address, 50H), which is the normal 8051 convention and matches the wording "starting at 50H".

- The final borrow in CY is 0, which confirms that the minuend was larger than the subtrahend, so the answer is a positive 24-bit number and no sign handling is needed.

- If the data were held in RAM rather than written as constants, only the addressing of the operands would change — for example SUBB A, 41H instead of SUBB A, #56H; the CLR C / SUBB / SUBB / SUBB structure would stay exactly the same.

Alternative version using a loop, for the case where the two numbers are already stored in RAM. The minuend is placed in 50H–52H (lowest byte first) so that the result replaces it in the required locations, and the subtrahend is placed in 40H–42H:

ORG 0000H ;---- load the two numbers into RAM, least significant byte first --- MOV 50H, #9CH ; minuend = 24759CH MOV 51H, #75H MOV 52H, #24H MOV 40H, #0F8H ; subtrahend = 0156F8H MOV 41H, #56H MOV 42H, #01H ;---- subtract three bytes in a loop ------------------------------- MOV R0, #50H ; pointer to the minuend / result MOV R1, #40H ; pointer to the subtrahend MOV R2, #03H ; number of bytes to process CLR C ; clear the initial borrow NEXT: MOV A, @R0 ; byte of the minuend MOV B, @R1 ; byte of the subtrahend SUBB A, B ; A = A - B - borrow MOV @R0, A ; store the result byte in place INC R0 ; INC and DJNZ do NOT disturb CY, INC R1 ; so the borrow survives into the DJNZ R2, NEXT ; next pass of the loop SJMP $ ; 50H = A4H, 51H = 1EH, 52H = 23H END

This version works for any number of bytes — only the constant in R2 has to be changed. It relies on an important property of the 8051: INC, MOV and DJNZ do not affect the carry flag, so the borrow generated by one SUBB is still intact when the next SUBB is executed.

Remember — only R0 and R1 can be used as indirect pointers on the 8051 (@R0, @R1). R2–R7 cannot. This is why the straight-line program of Step 3 is the correct and expected answer to this question.

# Q.5   Draw and explain the 8085 architecture

The Intel 8085 is an 8-bit, NMOS, single-chip microprocessor housed in a 40-pin dual-in-line package. It has an 8-bit data bus, a 16-bit address bus (so it can address 64 KB of memory and 256 input/output ports), works from a single +5 V supply and runs at a maximum internal clock of about 3 MHz (a 6 MHz crystal is divided by two on-chip). Its internal organisation is shown in Fig. 1.

Fig. 1 — Functional block diagram of the Intel 8085 microprocessor

## Explanation of the blocks

### 1.  Arithmetic and Logic Unit (ALU)

The ALU is the 8-bit computing unit of the processor. It performs addition, subtraction, increment, decrement, AND, OR, exclusive-OR, complement, rotate and compare operations. It always takes two operands — one from the accumulator and the other from the temporary register — performs the operation, places the result back in the accumulator, and records the nature of the result in the flag flip-flops. The ALU has no storage of its own; it is purely combinational logic.

### 2.  Accumulator (register A)

An 8-bit register which is physically part of the ALU. It is the most important register of the 8085 because:

- one operand of almost every arithmetic and logical operation must be in the accumulator, and the result is returned to it;

- all input/output instructions (IN and OUT) transfer data only through the accumulator;

- together with the flag register it forms the Program Status Word (PSW), which is what PUSH PSW and POP PSW save and restore.

### 3.  Temporary registers — Temp, W and Z

These 8-bit registers are not accessible to the programmer. The temporary register holds the second operand for the ALU while an operation is in progress. The W and Z registers hold intermediate bytes — for example the 16-bit address of a JMP, CALL or LXI instruction while the remaining bytes are still being fetched. They exist purely to make multi-byte instructions possible.

### 4.  Flag register (flag flip-flops)

Five flip-flops arranged as an 8-bit register, which record the status of the result produced by the ALU. Three bit positions are unused.

| Bit | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flag | S | Z | — | AC | — | P | — | CY |

| Flag | Name | It is set (= 1) when |
| --- | --- | --- |
| S | Sign | The MSB (bit 7) of the result is 1, i.e. the result is negative in signed arithmetic. |
| Z | Zero | The result of the operation is exactly 00H. This is the most frequently tested flag. |
| AC | Auxiliary Carry | A carry is generated out of bit 3 into bit 4. It is used only by the DAA instruction for BCD arithmetic and cannot be tested by any conditional jump. |
| P | Parity | The result contains an even number of 1s (even parity). |
| CY | Carry | An addition produces a carry out of bit 7, a subtraction needs a borrow, or a rotate instruction shifts a bit out. |

### 5.  Instruction register and instruction decoder

The first byte of every instruction fetched from memory (the opcode) is latched into the 8-bit instruction register. The instruction decoder decodes this opcode, and the machine-cycle encoder then determines how many machine cycles and T-states the instruction needs and what internal operations must be performed in each of them. The instruction register is not available to the programmer.

### 6.  Timing and control unit

This is the "nervous system" of the processor. Working in step with the clock, it generates every signal needed to execute the decoded instruction, both internally and on the pins:

| Group | Signals | Function |
| --- | --- | --- |
| Control | ALE, RD, WR, IO/M (RD, WR, IO/M are active low / status) | ALE latches the lower address byte externally; RD and WR are the read and write strobes; IO/M distinguishes a memory access from an I/O access. |
| Status | S0, S1 | Together with IO/M they identify the type of machine cycle (opcode fetch, memory read, memory write, I/O read, I/O write). |
| DMA | HOLD, HLDA | Allow an external device (a DMA controller) to request the buses; the 8085 floats its buses and acknowledges with HLDA. |
| Reset | RESET IN, RESET OUT | RESET IN clears the PC to 0000H and resets the interrupt enable and HLDA flip-flops; RESET OUT tells the rest of the system that the processor has been reset. |
| Clock | X1, X2, CLK OUT | X1 and X2 connect the crystal to the on-chip clock generator; CLK OUT supplies the system clock to the other chips. |
| Handshake | READY | A slow memory or I/O device pulls READY low to insert WAIT states until it is ready. |

### 7.  Register array (general-purpose registers)

Six programmer-accessible 8-bit registers — B, C, D, E, H and L. They can be used individually to hold 8-bit data, or in the fixed pairs BC, DE and HL to hold 16-bit quantities such as addresses and counters. The HL pair has a special status: it is the only pair that can be used as a memory pointer, and that pointer is written as the operand M in instructions such as MOV A, M, ADD M and INR M. The W and Z temporary registers also form part of this array but are internal only.

### 8.  Stack pointer (SP)

A 16-bit register that holds the address of the top of the stack, which is an area of read/write memory reserved by the programmer (normally with LXI SP, addr). The 8085 stack grows downwards: the SP is decremented before each byte is pushed by a PUSH or CALL, and incremented after each byte is popped by a POP or RET.

### 9.  Program counter (PC)

A 16-bit register that holds the address of the next instruction byte to be fetched. It is incremented automatically after every byte is fetched, so the program executes sequentially, and it is reloaded by the branching instructions (JMP, CALL, RET, RST) and by an interrupt. Its 16-bit width is what gives the 8085 its 64 KB memory range, and it is cleared to 0000H by RESET.

### 10.  Incrementer / decrementer address latch

A 16-bit unit that can add one to, or subtract one from, the contents of the PC, the SP or any register pair — it is this block that actually executes INX, DCX, the automatic increment of the PC and the automatic adjustment of the SP. It also latches the 16-bit address on its way from the register array to the address buffers.

### 11.  Address buffer and address / data buffer

The 8085 multiplexes the lower byte of the address with the data byte in order to save pins:

- The address buffer is a unidirectional 8-bit buffer that drives the higher-order address lines A15 – A8. These lines carry a stable address throughout the machine cycle.

- The address/data buffer is a bidirectional 8-bit buffer that drives the multiplexed lines AD7 – AD0. During the first T-state (T1) of every machine cycle these lines carry the lower byte of the address, which must be latched externally by an 8-bit latch (typically a 74LS373) using the ALE signal. During T2 and T3 the same lines carry the data.

This multiplexing is the single most characteristic feature of the 8085 hardware — it reduces the pin count from 48 to 40, but it makes an external address latch compulsory in every 8085 system.

### 12.  Interrupt control

The 8085 provides five hardware interrupt inputs with a fixed priority, plus one acknowledge output.

| Interrupt | Priority | Maskable? | Vectored? | Vector address | Triggering |
| --- | --- | --- | --- | --- | --- |
| TRAP | 1 (highest) | No (non-maskable) | Yes | 0024H | Edge and level sensitive |
| RST 7.5 | 2 | Yes | Yes | 003CH | Rising edge (latched) |
| RST 6.5 | 3 | Yes | Yes | 0034H | Level sensitive |
| RST 5.5 | 4 | Yes | Yes | 002CH | Level sensitive |
| INTR | 5 (lowest) | Yes | No | Supplied by the device | Level sensitive |

The maskable interrupts are controlled by the SIM (Set Interrupt Mask) and RIM (Read Interrupt Mask) instructions, and by EI/DI. When INTR is recognised the processor issues INTA (active low) so that the interrupting device can place an instruction — normally an RST n or a CALL — on the data bus.

### 13.  Serial I/O control

Two pins, SID (serial input data) and SOD (serial output data), allow simple one-bit-at-a-time serial transfer without any extra hardware. They are not a UART: the transfer is carried out entirely in software, the RIM instruction reading the SID pin into bit 7 of the accumulator and the SIM instruction writing bit 7 of the accumulator to the SOD pin.

### 14.  Internal buses

An 8-bit internal data bus interconnects the accumulator, the temporary register, the flag register, the instruction register, the register array and the buffers — only one of these may drive the bus at any instant, and the timing and control unit decides which. A separate 16-bit internal address bus carries addresses from the register array, the PC and the SP to the incrementer/decrementer latch and the address buffers.

## Salient features of the 8085 — summary

| Feature | Specification |
| --- | --- |
| Word length / data bus | 8-bit (lower 8 bits multiplexed with the address) |
| Address bus | 16-bit → 64 KB of memory and 256 I/O ports |
| Registers | A, B, C, D, E, H, L (8-bit) + PC, SP (16-bit) + flag register |
| Flags | Five — S, Z, AC, P, CY |
| Instruction set | 74 instruction types, 246 opcodes, 1 to 3 bytes long |
| Addressing modes | Five — immediate, direct, register, register indirect, implied |
| Clock | On-chip clock generator; 6.144 MHz crystal → ~3.07 MHz internal clock |
| Machine cycle | 3 to 6 T-states; instructions take 4 to 18 T-states |
| Interrupts | Five hardware interrupts (TRAP, RST 7.5 / 6.5 / 5.5, INTR) + 8 software RST instructions |
| Serial I/O | SID and SOD pins, handled by RIM and SIM |
| DMA | Supported through HOLD and HLDA |
| Technology / package / supply | NMOS · 40-pin DIP · single +5 V |

# Q.6   Describe the status register of the AVR

The Status Register (SREG) of the AVR is the 8-bit register that holds the result information of the most recently executed arithmetic or logical instruction, together with the global interrupt enable bit. It is updated after every ALU operation, and it is what makes conditional branching possible: a branch instruction simply tests one bit of SREG.

- It is an I/O register located at I/O address 0x3F, which corresponds to data-memory address 0x5F in devices such as the ATmega32 (the I/O space is mapped at 0x20 to 0x5F in the data space).

- It is accessed with the I/O instructions IN R16, SREG and OUT SREG, R16, and each of its bits also has a dedicated set/clear instruction.

- It is not automatically saved when an interrupt is taken — this is a very important practical difference from many other processors.

## Bit layout of SREG

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | I | T | H | S | V | N | Z | C |
| Read / Write | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
| Initial value | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

## Description of each bit

| Bit | Symbol | Name | Description |
| --- | --- | --- | --- |
| 7 | I | Global Interrupt Enable | Must be 1 before any interrupt can be serviced; when it is 0 all interrupts are disabled regardless of the individual enable bits. It is set by SEI and cleared by CLI. The hardware clears I automatically when an interrupt is accepted (so interrupts do not nest by default) and RETI sets it again on return. |
| 6 | T | Bit Copy Storage | A single-bit scratch store used by the bit-copy instructions: BST Rd, b copies bit b of register Rd into T, and BLD Rd, b copies T into bit b of Rd. It takes no part in arithmetic. |
| 5 | H | Half Carry Flag | Set when there is a carry out of bit 3 into bit 4 (that is, a carry between the two nibbles). It is required for BCD (packed decimal) arithmetic, where each nibble represents one decimal digit. |
| 4 | S | Sign Flag | Always equal to the exclusive-OR of N and V (S = N ⊕ V). It gives the true sign of a signed result even when an overflow has occurred, and it is the flag tested by the signed branches BRLT and BRGE. |
| 3 | V | Two's Complement Overflow Flag | Set when the result of a signed operation falls outside the representable range −128 to +127, for example when adding two positive numbers gives a negative answer. |
| 2 | N | Negative Flag | A copy of bit 7 (the MSB) of the result. In signed arithmetic a 1 means the result is negative. |
| 1 | Z | Zero Flag | Set when the result of the operation is exactly zero (0x00). This is the most frequently tested flag — BREQ and BRNE test it, and CP / CPI use it for equality comparisons. |
| 0 | C | Carry Flag | Set by a carry out of the MSB in addition, by a borrow in subtraction and comparison, and by the bit shifted out in the shift and rotate instructions (LSL, LSR, ROL, ROR, ASR). |

## Instructions that act directly on SREG

| Bit | Set it | Clear it | Branch if set | Branch if cleared |
| --- | --- | --- | --- | --- |
| I | SEI | CLI | BRIE | BRID |
| T | SET | CLT | — (use BLD / BST) | — |
| H | SEH | CLH | BRHS | BRHC |
| S | SES | CLS | BRLT | BRGE |
| V | SEV | CLV | BRVS | BRVC |
| N | SEN | CLN | BRMI | BRPL |
| Z | SEZ | CLZ | BREQ | BRNE |
| C | SEC | CLC | BRCS (= BRLO) | BRCC (= BRSH) |

The generic instructions BSET s and BCLR s set or clear SREG bit number s, and BRBS / BRBC branch on any SREG bit given by number.

## Worked example

Adding 0x80 to 0x80 affects six of the eight flags at once, which makes it an ideal illustration:

LDI R16, 0x80 ; R16 = 1000 0000 = 128 unsigned ; = -128 signed ADD R16, R16 ; 0x80 + 0x80 = 0x100 ; only the lower 8 bits are kept: ; R16 = 0x00 Resulting flags : C = 1 a carry was produced out of bit 7 Z = 1 the 8-bit result is 0x00 N = 0 bit 7 of the result is 0 V = 1 signed overflow : (-128) + (-128) = -256, which is outside the range -128 .. +127 S = N xor V = 0 xor 1 = 1 (the true sign is negative) H = 0 no carry occurred from bit 3 into bit 4

A second, simpler example — using the flags for a comparison:

LDI R17, 0x05 LDI R18, 0x05 CP R17, R18 ; compare (R17 - R18 = 0), registers unchanged ; => Z = 1, C = 0, N = 0 BREQ SAME ; this branch IS taken, because Z = 1 ... SAME: ...

## Saving SREG inside an interrupt service routine

Because the AVR hardware does not save SREG when an interrupt occurs, any ISR that executes an instruction which changes the flags must preserve it explicitly, otherwise the interrupted program may take a wrong branch after it resumes:

ISR: PUSH R16 ; save the working register IN R16, SREG ; read the status register PUSH R16 ; save it on the stack ... ; body of the interrupt routine POP R16 ; recover the old SREG value OUT SREG, R16 ; restore it POP R16 ; restore the working register RETI ; return and set the I bit again

Summary — SREG is the single 8-bit register through which the AVR ALU reports on its results (C, Z, N, V, S, H), provides a scratch bit for bit manipulation (T) and holds the master switch for the whole interrupt system (I).

# Q.7   Write a program to transfer the contents of R0 of bank 0 to register R7 of bank 1

## Background — the four register banks of the 8051

The first 32 bytes of the 8051’s 128 bytes of internal RAM (addresses 00H to 1FH) are organised as four banks of eight registers each. Each bank contains the registers R0 to R7, and only one bank is active at a time.

| Bank | RS1 (PSW.4) | RS0 (PSW.3) | Address range | R0 is at | R7 is at |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 00H – 07H | 00H | 07H |
| 1 | 0 | 1 | 08H – 0FH | 08H | 0FH |
| 2 | 1 | 0 | 10H – 17H | 10H | 17H |
| 3 | 1 | 1 | 18H – 1FH | 18H | 1FH |

The active bank is chosen by the two bits RS1 (PSW.4) and RS0 (PSW.3) of the Program Status Word. After a reset RS1 = RS0 = 0, so bank 0 is active by default.

Therefore — R0 of bank 0 is the internal RAM location 00H, and R7 of bank 1 is the internal RAM location 0FH. The problem reduces to copying the byte at 00H into the location 0FH.

## Method 1 — using bank switching (the expected answer)

Here the registers are referred to by their names R0 and R7, so the correct bank must be selected before each access.

;-------------------------------------------------------------------- ; Transfer R0 of bank 0 -> R7 of bank 1 ;-------------------------------------------------------------------- ORG 0000H CLR PSW.4 ; RS1 = 0 CLR PSW.3 ; RS0 = 0 -> BANK 0 is now selected MOV A, R0 ; A <- R0 of bank 0 (location 00H) SETB PSW.3 ; RS0 = 1 CLR PSW.4 ; RS1 = 0 -> BANK 1 is now selected MOV R7, A ; R7 of bank 1 <- A (location 0FH) SJMP $ ; stop END

A slightly shorter way of writing the same thing, setting both bank-select bits in one instruction by writing to PSW directly:

ORG 0000H MOV PSW, #00H ; RS1 = 0, RS0 = 0 -> bank 0 MOV A, R0 ; A <- R0 of bank 0 MOV PSW, #08H ; RS1 = 0, RS0 = 1 -> bank 1 MOV R7, A ; R7 of bank 1 <- A SJMP $ END

Caution — MOV PSW, #08H also clears the carry flag and the other PSW bits. When the flags must be preserved, use the bit instructions SETB PSW.3 / CLR PSW.3, which change only the one bit named.

## Method 2 — using direct addressing (shortest)

Every bank register also has a fixed direct address, and a register’s direct address is valid no matter which bank happens to be active. The transfer therefore needs no bank switching at all and can be done in a single instruction, using the MOV direct, direct form:

;-------------------------------------------------------------------- ; R7 of bank 1 (0FH) <- R0 of bank 0 (00H) ;-------------------------------------------------------------------- ORG 0000H MOV 0FH, 00H ; one instruction does the whole job SJMP $ END

Or, if the transfer is to go through the accumulator:

ORG 0000H MOV A, 00H ; A <- contents of 00H (= R0 of bank 0) MOV 0FH, A ; 0FH (= R7 of bank 1) <- A SJMP $ END

## Verification

| Location | Register it represents | Before execution | After execution |
| --- | --- | --- | --- |
| 00H | R0 of bank 0 | 45H | 45H (unchanged — it is a copy) |
| 0FH | R7 of bank 1 | xx (unknown) | 45H |

## Points to note

- The instruction is a copy, not an exchange: location 00H still holds its original value afterwards. A true exchange would need XCH A, 00H and XCH A, 0FH.

- In Method 1 the names R0 and R7 are interpreted relative to whichever bank is active at that moment — this is precisely why the bank must be switched between the two MOV instructions. Writing MOV A, R0 and MOV R7, A without switching would copy R0 to R7 within the same bank, which is not what was asked.

- Method 2 avoids the ambiguity completely, is shorter and faster, and leaves the PSW (and therefore the flags and the current bank) untouched. It is, however, less readable — a comment identifying 00H and 0FH is essential.

- The banks are a useful resource: switching banks gives an interrupt service routine its own private set of eight registers, which is much faster than pushing and popping R0–R7 on the stack.

- Because banks 1, 2 and 3 occupy 08H–1FH, a program that uses them must make sure the stack (which MOV SP, #… sets up) does not overlap that area — this is why SP is usually initialised above 1FH, or above 2FH if the bit-addressable region is also in use.

# Q.8   Write a brief note on the addressing modes of the AVR microcontroller

The addressing mode of an instruction is the way in which the instruction specifies where its operand is to be found. The AVR has a large, highly orthogonal instruction set and supports a rich set of addressing modes — this is one of the main reasons why the AVR is efficient for programs written in C.

Before the modes are listed, two features of the AVR register file should be recalled:

- There are 32 general-purpose 8-bit working registers, R0 to R31, all directly connected to the ALU, so most instructions complete in a single clock cycle.

- The last six of them form three 16-bit pointer registers:  X = R27:R26,  Y = R29:R28,  Z = R31:R30.  The Z pointer has the additional job of addressing the program memory.

## The addressing modes

### 1.  Register direct — single register (Rd)

The operand is in one register of the register file, and the result is written back into the same register.

INC R18 ; R18 <- R18 + 1 COM R20 ; R20 <- one's complement of R20 LSL R16 ; shift R16 left by one bit

### 2.  Register direct — two registers (Rd, Rr)

Both operands are in the register file; Rd supplies one operand and receives the result, Rr supplies the other and is unchanged.

ADD R16, R17 ; R16 <- R16 + R17 MOV R5, R10 ; R5 <- R10 (copy between registers) CP R18, R19 ; compare : flags only, nothing stored

### 3.  Immediate addressing

The operand is a constant contained in the instruction itself. Only the upper half of the register file, R16 to R31, can be used with these instructions.

LDI R16, 0x25 ; R16 <- 25H (load immediate) SUBI R20, 10 ; R20 <- R20 - 10 ANDI R18, 0x0F ; mask off the upper nibble CPI R17, 0x55 ; compare R17 with the constant 55H

### 4.  I/O direct addressing

Used to reach the 64 I/O registers (addresses 0x00 to 0x3F) — the ports, the timer registers, SREG and so on. The 6-bit I/O address is part of the instruction.

IN R16, PINB ; R16 <- the PINB input register OUT PORTB, R16 ; PORTB <- R16 IN R17, SREG ; read the status register

### 5.  Data direct addressing

A full 16-bit SRAM address is given in the instruction, so any location in the data space can be reached. These are two-word (32-bit) instructions and take two clock cycles.

LDS R16, 0x0100 ; R16 <- contents of SRAM address 0100H STS 0x0100, R16 ; SRAM address 0100H <- R16

### 6.  Data indirect addressing

The data address is supplied by one of the pointer registers X, Y or Z rather than by the instruction. This is the mode that makes arrays and pointers possible.

LDI R26, 0x00 ; set up X = 0x0100 LDI R27, 0x01 LD R16, X ; R16 <- contents of the location X points to ST Y, R18 ; the location Y points to <- R18

### 7.  Data indirect with displacement

The address is the pointer plus a 6-bit constant q (0 to 63) which is held in the instruction. The pointer itself is not changed. Only Y and Z can be used — X has no displacement form.

LDD R16, Y+5 ; R16 <- contents of (Y + 5) STD Z+3, R17 ; location (Z + 3) <- R17

Use: reaching the individual members of a structure, or a fixed element of an array, without disturbing the base pointer — exactly what a C compiler needs for struct access and for local variables on the stack frame.

### 8.  Data indirect with pre-decrement

The pointer is decremented first and the new value is then used as the address.

LD R16, -X ; X <- X - 1, then R16 <- contents of X ST -Y, R18 ; Y <- Y - 1, then store R18 there

### 9.  Data indirect with post-increment

The pointer is used first and is then incremented, so it is left pointing at the next byte.

LD R16, X+ ; R16 <- contents of X, then X <- X + 1 ST Z+, R0 ; store R0 at Z, then Z <- Z + 1 Copying a block of 16 bytes using post-increment : LDI R20, 16 LOOP: LD R16, X+ ; read and advance the source pointer ST Y+, R16 ; write and advance the destination pointer DEC R20 BRNE LOOP

Modes 8 and 9 together make the AVR very efficient at walking through buffers, strings and stacks.

### 10.  Program memory constant addressing (LPM)

Reads a constant stored in the Flash program memory — a look-up table, a message string, a set of seven-segment codes — using the Z pointer as the address. Z is a byte address, so its least significant bit chooses the low or the high byte of the 16-bit program word. ELPM is the extended form for devices with more than 64 KB of Flash.

LDI R30, LOW(2*TABLE) ; Z <- byte address of the table LDI R31, HIGH(2*TABLE) LPM R16, Z+ ; R16 <- the byte at Z, then Z <- Z+1 ... TABLE: .DB 0x3F, 0x06, 0x5B ; seven-segment codes in Flash

### 11.  Direct program addressing (JMP, CALL)

The complete absolute address (up to 22 bits, giving a 4 M-word reach) is contained in the two-word instruction, so anywhere in the program memory can be reached.

JMP 0x0100 ; PC <- 0x0100 CALL SUBROUTINE ; push PC, then PC <- the given address

### 12.  Indirect program addressing (IJMP, ICALL)

The target address is taken from the Z pointer:  PC ← Z.  This is how computed jumps and jump tables are implemented.

LDI R30, LOW(HANDLER) LDI R31, HIGH(HANDLER) IJMP ; PC <- Z (jump to the computed address) ICALL ; the same, but the return address is pushed

### 13.  Relative program addressing (RJMP, RCALL)

The instruction holds a signed 12-bit displacement k, and the new address is PC ← PC + k + 1 with k in the range −2048 to +2047 words. These are single-word instructions, and because the displacement is relative the code is position independent — it can be moved anywhere in the Flash without being reassembled.

RJMP LOOP ; relative jump, 1 word only RCALL DELAY ; relative call

### 14.  Relative addressing in conditional branches

All the conditional branches use a signed 7-bit displacement, so  PC ← PC + k + 1  with k in the range −64 to +63 words. The branch is taken only if the tested bit of SREG matches.

DEC R20 BRNE AGAIN ; branch if Z = 0 (R20 is not yet zero) BRCS OVERFLOW ; branch if C = 1 BRLT NEGATIVE ; branch if S = 1 (signed less-than)

### 15.  Bit (direct bit) addressing

A single bit of an I/O register or of a working register is set, cleared or tested, without disturbing the other seven bits. The SBI, CBI, SBIC and SBIS instructions work only on the lower 32 I/O registers (addresses 0x00 to 0x1F).

SBI DDRB, 3 ; make bit 3 of port B an output SBI PORTB, 3 ; drive PB3 high CBI PORTB, 3 ; drive PB3 low SBIC PINB, 0 ; skip the next instruction if PB0 = 0 SBRS R16, 7 ; skip the next instruction if bit 7 of R16 = 1 BST R16, 0 ; T <- bit 0 of R16 BLD R17, 4 ; bit 4 of R17 <- T

### 16.  Stack (implied) addressing

The address is supplied implicitly by the stack pointer (SPH:SPL); the instruction names no address at all. On the AVR the SP is post-decremented by PUSH and pre-incremented by POP, so the stack grows downwards in SRAM.

PUSH R16 ; store R16 at SP, then SP <- SP - 1 POP R16 ; SP <- SP + 1, then load R16 from SP

## Summary table

| # | Addressing mode | Typical instruction | Where the operand is |
| --- | --- | --- | --- |
| 1 | Register direct, one register | INC R18 | In a single register Rd |
| 2 | Register direct, two registers | ADD R16, R17 | In the two registers Rd and Rr |
| 3 | Immediate | LDI R16, 0x25 | A constant inside the instruction (R16–R31 only) |
| 4 | I/O direct | OUT PORTB, R16 | One of the 64 I/O registers (0x00–0x3F) |
| 5 | Data direct | LDS R16, 0x0100 | A 16-bit SRAM address inside the instruction |
| 6 | Data indirect | LD R16, X | The SRAM location the X / Y / Z pointer holds |
| 7 | Data indirect with displacement | LDD R16, Y+5 | Address = pointer + q, where q = 0…63 (Y, Z only) |
| 8 | Data indirect, pre-decrement | LD R16, −X | Pointer is decremented, then used |
| 9 | Data indirect, post-increment | LD R16, X+ | Pointer is used, then incremented |
| 10 | Program memory constant | LPM R16, Z+ | A constant in Flash, addressed by Z |
| 11 | Direct program addressing | JMP 0x0100 | A full absolute address in the instruction |
| 12 | Indirect program addressing | IJMP | The target address is in the Z pointer |
| 13 | Relative program addressing | RJMP LOOP | PC + k + 1, k = −2048…+2047 |
| 14 | Relative conditional branch | BRNE AGAIN | PC + k + 1, k = −64…+63 |
| 15 | Bit (direct bit) | SBI PORTB, 3 | One named bit of an I/O or working register |
| 16 | Stack (implied) | PUSH R16 | The location the stack pointer holds |

Why it matters — the combination of a large register file directly coupled to the ALU, three 16-bit pointers with automatic increment/decrement and displacement, and separate single-cycle bit instructions, is what allows the AVR to execute one instruction per clock cycle and makes it a particularly good target for a C compiler.

# Q.9   Explain in brief the hardware modules of the AVR with a suitable diagram

The AVR is a family of 8-bit microcontrollers from Atmel (now Microchip) built on a modified Harvard RISC architecture. A typical mid-range member such as the ATmega32 or ATmega328P contains, on a single chip, the CPU core, three kinds of memory, a full set of peripherals and all the system support circuits. These hardware modules are shown in Fig. 2 and described below.

Fig. 2 — Hardware modules of a typical AVR microcontroller (ATmega32 / ATmega328P)

## A.   The CPU core

The 8-bit RISC CPU is organised around a register file of 32 general-purpose 8-bit registers (R0–R31) which are connected directly to the ALU, so that two operands can be fetched, the operation performed and the result written back in a single clock cycle. The core uses a two-stage pipeline — while one instruction is being executed the next is already being fetched — which gives a throughput of up to 16 MIPS at 16 MHz, roughly ten times that of the 8051 at the same clock. The core contains:

- the Program Counter (PC), the instruction register and the instruction decoder;

- the ALU, which performs arithmetic, logic and bit operations, and includes a hardware multiplier (2-cycle MUL) in the ATmega devices;

- the Status Register (SREG) holding the I, T, H, S, V, N, Z and C flags;

- the Stack Pointer (SPH:SPL), which points into the internal SRAM, the stack growing downwards;

- the three 16-bit pointer registers X, Y and Z, formed from R26–R31.

## B.   The three memories

| Memory | Typical size | Type | What it holds |
| --- | --- | --- | --- |
| Flash program memory | 32 KB (16 K words) | Non-volatile, in-system self-programmable, ~10,000 write/erase cycles | The program itself, and constant look-up tables read with LPM. It has its own address space (this is the Harvard feature) and is organised in 16-bit words. An optional boot-loader section allows the chip to reprogram itself. |
| Internal SRAM | 2 KB | Volatile | Variables, arrays and the stack. The data space is one linear map which also contains the 32 registers (0x00–0x1F) and the 64 I/O registers (0x20–0x5F), with the SRAM starting at 0x60. |
| EEPROM | 1 KB | Non-volatile, byte addressable, ~100,000 cycles | Calibration constants, user settings, serial numbers and logged data that must survive a power cut. It is accessed through the EEAR, EEDR and EECR registers, not by ordinary LD/ST instructions. |

## C.   Input / output ports

The device has four 8-bit bidirectional ports — A, B, C and D (the ATmega328P has three). Every pin is individually configurable and bit-addressable through three registers:

| Register | Name | Function |
| --- | --- | --- |
| DDRx | Data Direction Register | A 1 makes the pin an output, a 0 makes it an input. |
| PORTx | Port Output Register | For an output pin it holds the value to be driven; for an input pin, writing a 1 switches on the internal pull-up resistor. |
| PINx | Port Input Pins | Read-only — it returns the actual logic level present on the pin. |

Each pin can source or sink up to about 20 mA, which is enough to drive an LED directly, and almost every pin also carries an alternate function belonging to one of the peripherals (ADC input, PWM output, serial line, external interrupt, and so on).

## D.   Timer / counter modules

- Timer/Counter 0 — 8-bit: normal counting, clear-timer-on-compare (CTC), fast and phase-correct PWM, with a prescaler (1, 8, 64, 256, 1024).

- Timer/Counter 1 — 16-bit: the most capable timer. It adds input capture (for measuring pulse widths and frequency) and two independent output-compare channels, giving two PWM outputs with up to 16-bit resolution.

- Timer/Counter 2 — 8-bit: can be clocked asynchronously from a separate 32.768 kHz crystal, which lets it run as a real-time clock while the rest of the chip is asleep.

All of them can generate interrupts on overflow and on compare match, and are used for delays, periodic tasks, frequency and pulse-width measurement, motor speed control and D/A conversion by PWM.

## E.   Analogue modules

- 10-bit ADC — a successive-approximation analogue-to-digital converter preceded by an 8-channel input multiplexer. The reference may be AVCC, an internal 2.56 V source or an external voltage on the AREF pin. It supports single-conversion and free-running modes, takes about 13 µs per conversion, and has a dedicated ADC noise-reduction sleep mode in which the CPU is stopped during the conversion to improve accuracy.

- Analogue comparator — compares the voltages on the AIN0 and AIN1 pins and can raise an interrupt when the result changes; useful for threshold detection and for zero-crossing detection.

## F.   Serial communication modules

| Module | Pins | Characteristics |
| --- | --- | --- |
| USART | RxD, TxD | Universal Synchronous/Asynchronous Receiver-Transmitter. Full duplex, programmable baud rate, 5 to 9 data bits, optional parity, double-buffered, with three separate interrupts (TX complete, RX complete, data register empty). Used for RS-232 / TTL serial links, GPS and GSM modules, and PC communication. |
| SPI | MOSI, MISO, SCK, SS | Serial Peripheral Interface — a fast synchronous master/slave link (up to half the system clock). Used for serial EEPROMs and Flash, SD cards, displays and radio modules, and also for in-system programming of the AVR itself. |
| TWI (I²C) | SDA, SCL | Two-Wire Interface, Atmel’s implementation of I²C. Byte-oriented, multi-master with arbitration, 7-bit addressing, up to 400 kHz. Allows up to 128 devices on just two wires — used for real-time clocks, sensors and EEPROMs. |

## G.   Interrupt unit

Each interrupt source — the external pins INT0, INT1 and INT2, every timer event, the ADC, the USART, the SPI, the TWI, the EEPROM and the analogue comparator — has its own vector in a fixed interrupt vector table at the bottom of the Flash. Priority is fixed by position in the table: the lower the vector address, the higher the priority. An interrupt is served only if its individual enable bit and the global I bit of SREG are both set. The vector table can optionally be moved to the boot section.

## H.   System support modules

- Clock and oscillator circuits — a calibrated internal RC oscillator (1, 2, 4 or 8 MHz) needing no external parts, an external crystal or ceramic resonator on XTAL1/XTAL2, a low-frequency 32.768 kHz crystal option, or an external clock. A programmable prescaler then distributes the clock separately to the CPU, the Flash, the I/O and the ADC.

- Watchdog timer — an independent counter running from its own on-chip oscillator. If the program does not clear it with the WDR instruction within the selected time-out (16 ms to 8 s), the watchdog resets the chip. This is the standard protection against a software hang.

- Reset and supervision — power-on reset, the external RESET pin, a brown-out detector with a selectable trip level that holds the chip in reset while the supply is too low, the watchdog reset and (where fitted) the JTAG reset.

- Power management — six software-selectable sleep modes (idle, ADC noise reduction, power-down, power-save, standby, extended standby) plus the ability to switch off the clock to individual peripherals. In power-down mode the current falls to a fraction of a microamp, which is what makes battery-powered AVR designs practical.

- JTAG / debugWIRE on-chip debug — an IEEE 1149.1 compliant JTAG interface providing boundary-scan testing, real-time on-chip source-level debugging with breakpoints, and programming of the Flash, EEPROM, fuses and lock bits. Smaller devices use the single-wire debugWIRE interface instead.

- Programming interface, fuses and lock bits — the Flash and EEPROM can be programmed through SPI/ISP serial programming, through JTAG, or by a boot loader. The fuse bits permanently configure options such as the clock source, the brown-out level and the boot behaviour, while the lock bits can prevent the program memory from being read out, protecting the design.

## Summary of the modules

| Group | Modules |
| --- | --- |
| CPU core | 32 × 8 register file · ALU with hardware multiplier · SREG · PC · instruction decoder · stack pointer · 2-stage pipeline |
| Memory | 32 KB Flash (program) · 2 KB SRAM (data) · 1 KB EEPROM (non-volatile data) |
| Digital I/O | Four 8-bit bidirectional ports A–D, each pin individually configurable (DDRx, PORTx, PINx) |
| Timing | Timer/Counter 0 (8-bit) · Timer/Counter 1 (16-bit, capture + compare) · Timer/Counter 2 (8-bit, asynchronous) · PWM channels |
| Analogue | 10-bit 8-channel ADC · analogue comparator · internal voltage reference |
| Serial | USART · SPI · TWI (I²C) |
| System | Interrupt unit · watchdog timer · oscillators and clock prescaler · power-on reset · brown-out detector · six sleep modes |
| Debug / programming | JTAG or debugWIRE on-chip debug and boundary scan · SPI/ISP serial programming · fuse and lock bits |

# Q.10   Compare AJMP, LJMP and SJMP in the microcontroller

SJMP, AJMP and LJMP are the three unconditional jump instructions of the 8051. All three do exactly the same thing — they load a new value into the program counter — and all three take the same 2 machine cycles. They differ only in how the target address is coded inside the instruction, and that difference decides how many bytes the instruction occupies and how far it can reach.

## Comparison table

| Point of comparison | SJMP | AJMP | LJMP |
| --- | --- | --- | --- |
| Full name | Short Jump | Absolute Jump | Long Jump |
| Size of the instruction | 2 bytes | 2 bytes | 3 bytes |
| Opcode | 80H, followed by the displacement | The three most significant bits of the target are packed into the opcode byte as a10 a9 a8 0 0 0 0 1, so the opcode is one of 01H, 21H, 41H, 61H, 81H, A1H, C1H or E1H | 02H, followed by the two address bytes |
| Operand written in the instruction | 8-bit signed relative displacement | 11-bit page address (addr11) | 16-bit absolute address (addr16) |
| Machine cycles | 2 | 2 | 2 |
| Effect on the PC | PC ← PC + 2 + rel (rel is treated as a signed byte, −128 … +127) | PC10–PC0 ← addr11; PC15–PC11 are left unchanged | PC ← addr16 (all sixteen bits are replaced) |
| Range it can reach | 256-byte window — from 128 bytes back to 127 bytes forward, measured from the instruction that follows the SJMP | Anywhere inside the current 2 KB page (2 KB = 2048 bytes, since 11 bits are supplied) | Anywhere in the full 64 KB of program memory |
| Position independent (relocatable)? | Yes — the code may be moved anywhere without reassembling, because the displacement is relative | No — the target must lie in the same 2 KB page as the following instruction | No — the absolute address is fixed in the code |
| Code memory used | Least | Least | Most (one extra byte) |
| Typical use | Small loops, skipping a few instructions, and SJMP $ to halt the program | Jumping about inside one 2 KB program module, saving a byte compared with LJMP | Jumping anywhere — above all LJMP MAIN at the reset vector 0000H |
| Example | SJMP AGAIN | AJMP NEXT | LJMP 0F200H |

## Worked illustration of the three forms

ORG 0000H LJMP MAIN ; 3 bytes - the reset vector has room for ; exactly 3 bytes before the first ; interrupt vector at 0003H ORG 0030H MAIN: MOV A, #00H AGAIN: INC A MOV P1, A SJMP AGAIN ; 2 bytes, relative : the assembler works ; out the displacement for itself AJMP ELSEWHERE ; 2 bytes : ELSEWHERE must be inside the ; same 2 KB page as the next instruction STOP: SJMP $ ; "$" means "this address" - the usual way ; of stopping an 8051 program

## Important points

- All three take the same time. Each is a 2-machine-cycle instruction, so choosing SJMP or AJMP over LJMP saves code memory, not execution time.

- The 2 KB page trap of AJMP. AJMP replaces only the lower 11 bits of the PC, so the "page" is determined by the upper 5 bits of the address of the instruction that follows the AJMP. If an AJMP sits near the end of a 2 KB boundary and its target lies just beyond it, the jump will land in the wrong page. The assembler reports this as a "target out of range" error — it cannot be fixed by moving the label, only by using LJMP.

- Why SJMP is preferred where it fits. Because the displacement is relative and computed by the assembler, a routine written with SJMP and conditional branches can be relocated anywhere in the Flash without change. This is why library and relocatable code uses SJMP wherever the ±127-byte range allows.

- `SJMP $` is the standard way to stop. The 8051 has no HLT instruction, so a program is ended with an infinite two-byte loop — written either as SJMP $ or as HERE: SJMP HERE.

- All the conditional jumps are relative. JZ, JNZ, JC, JNC, JB, JNB, JBC, CJNE and DJNZ all use an 8-bit signed displacement like SJMP, with the same ±128-byte limit. When a conditional jump must reach further, the usual trick is to branch over a nearby LJMP:

JNZ NEAR ; the conditional jump reaches only +-127 LJMP FAR_AWAY ; ... so jump over an LJMP that reaches 64 KB NEAR: ...

- The call instructions follow the same pattern. ACALL is the 2-byte, 11-bit (2 KB page) counterpart of AJMP, and LCALL is the 3-byte, 16-bit counterpart of LJMP. There is no "SCALL" — the 8051 has no relative call instruction.

- A fourth, indirect jump exists. JMP @A+DPTR computes its target at run time (PC ← A + DPTR) and is used for jump tables and state machines; it is not one of the three being compared here because its address is not written into the instruction at all.

Conclusion — use SJMP when the target is within ±127 bytes (smallest, fastest to write, relocatable), AJMP when it is within the same 2 KB page and a byte of code memory is worth saving, and LJMP when the target may be anywhere in the 64 KB code space — which is why the reset vector at 0000H almost always contains LJMP MAIN.